Evaluation of two remediation techniques applied to a site impacted by petroleum production waters

Evaluación de dos técnicas de remediación aplicadas a un sitio impactado por aguas de producción petrolera

Ambar Y. Gómez-Mellado1, Carlos M. Morales-Bautista1‡, Iliana M. De la Garza-Rodríguez2, Sonia A. Torres-Sánchez3, Irma Sánchez-Lombardo1

1Grupo de Investigación en Ciencias de la Tierra. División Académica de Ciencias Básicas. Universidad Juárez Autónoma de Tabasco. Carretera Cunduacán-Jalpa KM. 1 Col. La Esmeralda CP. 86690. Cunduacán, Tabasco, México.

2Facultad de Ciencias Químicas. Universidad Autónoma de Coahuila. Blvd. V. Carranza s/n esq. con Ing. José Cárdenas Valdés. Col. República Ote. C.P. 25280. Saltillo, Coahuila, México.

3Facultad de Ingeniería. Universidad Autónoma de San Luis Potosí. Álvaro Obregón #64, Col. Centro, C.P. 78000. San Luis Potosí, S.L.P. México.

‡Author responsable (carlos.morales@ujat.mx)

SUMMARY

Soil salinity is a problem that has increased in recent years, in the southeast of Mexico one of the main sources associated with these effects is the water of oil production, called congenital water. Although the chemical composition varies from site to site, some studies have shown that its components can cause harmful effects on health and ecosystems. To recover the vocation of use of the affected sites, different treatments have been established that only remove parameters that are within the norm without taking into account other phenomena that may influence the quality of the soil. For this, two techniques of treatment of a soil experimentally contaminated by congenital water were evaluated, these were: Cation exchange and natural attenuation, the evaluation consisted in comparing the physical and chemical properties of control, treated and treated soil after one year, also the metal content in these latters was determined. In the results it was observed that congenital water causes changes in soils and that, to remove salinity, cation exchange is more effective than short-term natural attenuation, but does not recover texture, densities, porosity and organic matter, in comparison with natural attenuation, being this treatment more feasible in the long term; metals were also found in both cases but below the regulatory limit, however, long-term studies must be carried out since the area is constantly immersed in spills and emissions derived from the oil industry.

Index words: natural attenuation, cation exchange, vocation of use, impacted site.

RESUMEN

La salinidad en suelos es un problema que se ha incrementado en los últimos años, en el sureste mexicano una de las principales fuentes asociadas a estos efectos es el agua de producción petrolera, denominada agua congénita. Aunque la composición química varía según cada sitio, algunos estudios han mostrado que sus componentes pueden causar efectos nocivos sobre la salud y los ecosistemas. Para recuperar la vocación de uso de los sitios afectados, se han establecido diferentes tratamientos que solo remueven parámetros que están dentro de la norma sin tomar en cuenta otros fenómenos que pueden influir en la calidad del suelo. Por esto, se evaluaron dos técnicas de tratamiento de un suelo contaminado experimentalmente por agua congénita, estas fueron; Intercambio Catiónico y Atenuación Natural, la evaluación consistió en comparar las propiedades físicas y químicas de suelo testigo, tratado y tratado después de un año, del mismo modo, se determinó el contenido de metales en estos últimos. En los resultados se observó que el agua congénita provoca cambios en los suelos y que, para remover la salinidad, el intercambio catiónico es más efectivo que la atenuación natural a corto plazo, pero no recupera textura, densidades, porosidad y materia orgánica, en comparación con atenuación natural, siendo este tratamiento más factible a largo plazo, también en ambos casos se encontró metales pero debajo del límite normativo, no obstante, deben de realizarse estudios a largo plazo ya que la zona se encuentra inmersa constantemente en derrames y emisiones derivados de la industria petrolera.

Palabras clave: atenuación natural, intercambio catiónico, vocación de uso, sitio impactado.

INTRODUCTION

Several studies have shown the importance of soil conservation for the species that inhabit it since they obtain the nutrients necessary for its full development, however, these conditions can be altered by the discharge of external agents from the environment. Due to the magnitude of the associated effects, the anthropogenic sources are the ones that they consider of highest important (Porta et al., 2003). Generally, it has been observed that during the processes of manufacturing and use of natural resources, accidents occur in which spills of components succend that can interrupt physical, chemical and biological processes of soil, for these reasons, the fertility of a site it is lost and, consequently, there is low access to fooding or negative effects on public health (Becerra-Castro et al., 2015). Also, there are other investigations that have documented negative impacts in areas surrounding facilities of the different productive sectors, many of them have not been served for a long time and, in the case of soils, the dimension of the effects depends on the conditions environmental of each particular site (Adams et al., 2008; Labra-Cardón et al., 2012; Cuero, 2013).

In some of these regions, the most representative source of pollution are the derivatives and residues of the oil industry, of which, crude oil, fuels and water of formation or production called congenital water stand out, due to the composition they possess, different studies they are considered as dangerous, in some cases, below by normative limits (Khodaveisi et al., 2011; Cavazos-Arroyo et al., 2014; Ordóñez et al., 2014). The reports of the emergencies of these spills mention that most of them occur due to ruptured pipelines related to the lack of maintenance or natural disasters, but in some countries, there has recently been an increasing tendency that these are provoked, mainly during the fuel theft (Romo, 2016; Enamorado et al., 2016; Oswald, 2017). To remedy these sites, different proposals have been carried out, some have managed to re-establish the vocation of use, despite this, there are still problems after restoration such as toxicity and water repellency (Fawzy, 2008; Yongming, 2009; Arao et al., 2010). In this sense, several investigations have shown the importance of evaluating the bioaccumulation of metals and aromatic compounds after remediation work and have emphasized that the behavior of the pollutant depends on the specific conditions of each region, in some cases, the contaminants can be transferred in the middle and in this way reach the human being, there are also sites in which the hydrocarbons are below the regulatory limit and have negative impacts on the development of some plants, while others are above this limit, they do not present affectations; different researchers have emphasized that in order to restore a site, different alternatives must be evaluated and the long-term efficiency of each of them must be compared, in such a way that the information generated can serve as a decision-making tool to restore the properties of the site, optimally, feasibly and adequately (Liang et al., 2011; Hou & Al-Tabbaa, 2014; Bolan et al., 2014; Marín-García et al., 2016, Lu et al., 2017).

In Mexico, until date no real monitoring to soil pollution, it is estimated that the annual deterioration has impacted around 64% of the surface of the national territory due to various processes, chemical degradation being an important factor, since it is considered as contamination by action of residues of chemical compoust, many of them classified as dangerous according to NOM-052-SEMARNAT-2005, it is important to note that some of the residues of the oil industry are within this standard (SEMARNAT, 2005; Riojas-Rodríguez et al., 2013; Surendra et al., 2014). For this sector, until a few years ago who established the statutes to determine environmental impacts was the Secretary of Environment and Natural Resources (SEMARNAT), but after the law reform in energy matters in 2013, the Safety, Energy Agency was created and Environment (ASEA), currently this body has established guidelines to diagnose negative effects depending on the activity to be performed, however, only certain areas of interest have been studied, mainly those where there is private capital investment, in general, in the reports presented as "Environmental Base Line", some sites have been declared with presence of hydrocarbons and heavy metals, but there are no records of what are or will be the mitigation measures for each zone (Mendoza & Ize, 2017; Lázaro, 2017).

For the southeast region of this country, one of the pollutants that has been little studied is congenital water, which usually varies its components according to the source of generation and final disposal, therefore, also effects it has on soils (Li et al., 2016). This waste, generated in dehydration plants or separation batteries, contains hydrocarbons in low concentrations, solids in suspension, heavy metals (mainly chromium and vanadium), high electrical conductivity associated with sodium precursor salts, whose concentration is usually variable (between 180 000 and 300 000 mg·L-1), all these parameters are governed by NOM 143 SEMARNAT-2003 for final disposal (SEMARNAT, 2003). However, for various reasons spills of this waste have been caused during its transport, affecting soil and water, especially in the states of Tabasco and Veracruz (Morales-Bautista et al., 2011; Ortiz-Salinas et al., 2012). To remedy the affected areas has been taken as a basis salinity, the literature 6states that this occurs when there are excessive accumulations of sodium precursors in living tissues, however, there are other metals that are usually present in the contaminant and also are deposited on the ground, for example, in the case of plants, some reports suggest that the response depends not only on the concentration but also on the composition of the salts present and the properties of the soil, even some species may be tolerant to these conditions and allow  bioaccumulate the pollutants, in several cases of studies, it is mentioned that many of these species are used in some sites as an alternative for remediation, however, some problems have occurred in which the risk to health increases because the restored areas are used for agricultural or cattle raising, mainly due to ignorance of the population (Mulligan et al., 2001; Ruyters et al., 2011: Yao et al., 2012).

The majority of soil treatment techniques base their objectives according to the risk represented by the pollutant, for soil salined, in Mexico the criteria of NOM-021-SEMARNAT-2000 are used, most of them have managed to restore the main indicators to regulaty levels , however, several studies show that other properties such as pH, organic matter (% MO) and field capacity (% CC) are determinants in the survival and development of a plant, as well as the metals present that can be absorbed (SEMARNAT , 2002, Jayasekera & Hall, 2007; Prieto et al., 2009; Dastgheib et al., 2011) Different lines of research agree that the relevance of studying each variable involved in the remediation systems will allow knowing the level of risk of each component and, in this way, to be able to determine the measures for the prevention of impacts when they are significant (Kadali et al., 2012; Morales-Bautista et al., 2016). there are methods to eliminate the most dangerous part of the pollutant, while others are based on stabilization and as a result the disposition to the environment is inhibited, consequently, its toxicity; All of the above can be carried out through biological, physical and chemical processes. When compared to each other, not only the cost-benefit of each of them can be evaluated, also the enviorental feasiblility (Peng et al., 2009; Saint-Laurent et al., 2010).

Due to the above, this paper addresses two techniques of remediation of soils affected by salinity, considering the effects of environmental conditions in tropical areas which play an important role in the behavior of soils, mainly, alluvial. These technologies are cation exchange (IC) and natural attenuation (AN) (Lakhdar et al., 2009; Tang et al., 2012; Etim, 2012; Song et al., 2017). Within the study, the soil was experimentally contaminated with congenital water, treated by these alternatives and subjected to one year of weathering in order to observe whether the variations of rainfall and temperature of the Tabasco state influence after the treatment process. As response variables, the physical and chemical properties of the control, contaminated and treated soils were evaluated, in the same way, the content of metals in congenital water and in treated soil after a period of expose to weathering.

MATERIALS AND METHODS

A directed sampling was carried out in a land located between the Samaria II Separation Battery and the Samaria river margin, the facilities belong Petróleos Mexicanos (PEMEX, exploration and production) within the municipality of Cunduacán, Tabasco, Mexico; Although the industrial development is evident, the zone still conserves the vocation of agricultural-livestock (Ruíz-Álvarez et al., 2012); for this reason, in the margins of the oil infrastructure the inhabitants develop activities such as grazing, sowing and raising backyard animals, mostly for self-consumption (Gutiérrez & Zavala, 2002; Ruíz-Álvarez et al., 2012; Pinkus-Rendón & Contreras -Sánchez, 2012; Maldonado-Sánchez et al., 2016).

During the sampling, fodder grass "chontalpo" (Brachiaria brizantha) and trees used as a live fence, mainly of "cocoite" (Gliricidia sepium), could also be identified, and some poultry and fetting cattle were also observed. The sampling was carried out using shovels and steel nucleators, three composite geological strata were taken, which were collected from the rhizosphere to the water table (70-80 cm), the locations of the points are denoted in the Table 1.

Table 1. Reference of samples of control soil.

| Point | Location UTM 15 Q ± 3 m |  |  |
| --- | --- | --- | --- |
| 1 | 484 513.54 E | 1 996 107.86 N | 9 msnm |
| 2 | 484 388.87 E | 1 996 079.81 N | 11 msnm |
| 3 | 484 530.05 E | 1 995 961.33 N | 8 msnm |

UTM = Universal Transverse Mercator; msnm = meters above sea level.

In general, the samples showed sand-clayey texture to the touch, medium humidity, high reaction to hydrogen peroxide and medium to acetic acid, colour orange mottles were observed (7.5 YR 10/8) and soil base was identified as color light-brown (10 YR 4/4) (according to the Munsell color chart), few deep roots (25-20 cm) but abundant, as well as earthworms were observed. Due to the horizons being homogenized in most of the soil remediation tasks, in the present work, composite (homogenized) samples were always analyzed. All the extracts were placed on a canvas and homogenized with shovels, a quartering was performed and three samples of approximately 3 kg each were taken, these were labeled as control soil, kept in black and fridge with ice, the rest was stored in bags and, in this way, all were transferred to the laboratory where they were treated separately. On the one hand, the witnesses were dried in a stove with a stable atmosphere, roots and rocks were removed, and finally, they were ground and sieved, instead, those of the sacks, only roots and rocks were removed, manually homogenized sample and later thhen they were placed in plastic containers with holes in the bottom to which they were named experimental units (a total of nine with approximately 10 kg of soil in each one) and left to rest by 10 days under roof.

After this time, six of the units were contaminated experimentally with approximately 15 liters of congenital water from the Samaria Battery II (UTM 15Q, 490531E, 1989895 N), it was left to rest for 10 more days and take one sample homogenized of 500 g of each were, they all joined and homogenized a quartering was made and three samples of 2 kg were taken, these were named contaminated soil.

Three of the contaminated units were treated by cationic exchange with CaO, for this 0.5 mole of Ca was added for every 1 mole of excess Na in the contaminated soil with respect to the control. Was added The treating agent to the contaminated soil, with spatula and cubetts, the is homogenized and make washings with potable water, these were distributed in three amendments of five liters, one per day) (Morales-Bautista et al., 2011; Morales-Bautista et al., 2016). The other three, were remediated by biological method adding potable water washes (10 liters distributed in two days), subsecuently, added organic materia (4% by weight with respect to contaminated soil and, with composition of 13% Nitrogen, 2% Phosphorus, 44% Potassium) homogenizing with a spatulaand, finally, one last water amendment (five liters) (Ebuehi et al., 2005; Tang et al., 2012). The treated soils were allowed to stand for 10 days for the water to drain; after this time, they were homogenized and from each cation exchange cell 1 kg were taken and placed on a canvas, homogenized, a quartering was made to extract three composite samples of approximately 1 kg, these were called fresh cation exchange ( ICF), all the previous, was also made for the units tratated for biological method and, these were called fresh natural attenuation (ANF).

The experimental design is shown in Table 2, the parameters to be determined are shown in Table 3, in the same way, Table 4 specifies the weathering conditions to which the treated soils were subjected, to evaluate, all were analyzed by ANOVA (Boqué & Maroto, 2004; Hurtado & Silvente, 2012).

Table 2. Experimental design.

| Experimental design |  |  |  |
| --- | --- | --- | --- |
| Contaminated | R1 | R2 | R3 |
| Treatment 1 (IC) | R1 | R2 | R3 |
| Treatment 2 (AT) | R1 | R2 | R3 |

R1, R2 and R3: repetitions.

Table 3. Parameters to be determined in soils.

| Soil |  | Metals |  |
| --- | --- | --- | --- |
| pH |  | Lead | Pb |
| CE | dS·m-1 | Nickel | Ni |
| CC | % | Zinc | Zn |
| Texture | % | Cadmium | Cd |
| DA | g·cm-3 | Chrome | Cr |
| DR | g·cm-3 | Sodium | Na |
| Po | % | Potassium | K |
| CIC | Cmol+·kg-1 | Calcium | Ca |
| MO | % | Iron | Fe |
|  |  | Magnesium | Mg |
|  |  | Vanadium | V |
|  |  | Copper | Cu |

CE: Electrical Conductivity; CC: Field Capacity; Textures: Arenas (% A), Clays (% R) and Limo (% L); DA: Apparent density; DR: Real Density; % Po: Porosity; CIC: Cationic Exchange Capacity and, % MO: Organic matter.

Table 4. Environmental conditions in weathering period (CNA, 2019).

| Month | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T | 22.1 | 25.8 | 27.5 | 28.3 | 29.3 | 29 | 29.4 | 28.5 | 28.6 | 27.9 | 26.2 | 24.3 |
| Pr | 323.9 | 58.3 | 49.3 | 109.5 | 59.5 | 129.9 | 109.5 | 195.7 | 218.1 | 253.7 | 358.8 | 99.4 |

T is the average monthly temperature in ° C; Pr is the average monthly rainfall in mm.

RESULTS AND DISCUSSION

In Table 5 present the parameters determined to the congenital water of the Samaria II battery are presented according to the methods of NOM-143-SEMARNAT-2003 (SEMARNAT, 2003).

Table 5. Characterization of congenital wáter of Battery of separation Samaria II.

| Component | Valuel | Metal | Concentration (mg·L-1) |
| --- | --- | --- | --- |
| Total Solids | 101 mg·L-1 | Pb | 0.3 |
| Dissolved solids | 95% | Ni | 0.075 |
| Sedimentable solids | 1% | Zn | 0.146 |
| Total suspended soils | 4% | Cd | <LD |
| HTP | 10 mg·L-1 | Cr | <LD |
| pH | 6 | Na | 6639 |
| DBO | 9 mg·L-1 | K | 128.9 |
| CE | 140.14 dS·m-1 | Ca | 48.2 |
| Colour | 550 UPt·Co-1 | Fe | 0.877 |
| CaCO3 | 420 mg·L-1 | V | 0.059 |
| Total Coliforms | 990 NMP | Mg | 38.475 |
| Total Chorides | 40 mg·L-1 | Cu | <LD |

HTP: Total Petroleum Hydrocarbons, COD: Biochemical Oxygen Demand, CE: Electrical conductivity, UPt / Co: Units in platinum/cobalt and NMP scale: Probable number and L.D: detection limit.

According to the results obtained from the analyzes carried out on congenital water, it is observed that the values ​​of the components are below the maximum permissible limit (LMP) for discharge in coastal waters as established by NOM-143-SEMARNAT-2003 , however, when comparing with the limits of NOM-001-SEMARNAT-1996 for discharges in this same source, it was detected that some parameters are above the LMP, these are floating matter corresponding to the suspended solids and Pb, the latter considering daily discharge. Several studies have shown that one of the problems of metals is that, although they can be in low concentrations and adequate comprehensive management is carried out through various treatments to reach regulatory,  are bioaccumulative (Diya'uddeen et al., 2011); several reports show that in sites exposed to spill conditions or emissions of these compounds, the population is in constant risk, for example, there are jobs that mention that when dumped into the sea the species that develop there can fix metals, so the toxicity in the whole trophic chain is always latent by the biomagnification (Azetsu-Scott et al., 2007; Verma & Dwivedi, 2013; Hobman & Crossman, 2015). Also, high values ​​were observed for CE and sodium, which is why it is considered that if this waste is spilled in the soil it can be associated with salinity problems, although some reports have emphasized that these effects depend on the amount spilled and the type of water. soil (Qin et al., 2012; Li et al., 2016).

On the other hand, in Table 6 presents the results of the physical and chemical properties of the control soils, contaminated, treated and weathered both by ion exchange and by environmental attenuation.

Table 6. Physical and chemical parameters of control, contaminated, treated and weathered soils.

| Soil | pH | CE | CC | %A | %L | %R | DA | DR | %Po | %MO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T | 7.20 ± 0.09 | 0.13 ± 0.002 | 35 ± 1.11 | 55 ± 2.01 | 10 ± 0.11 | 35 ± 0.61 | 1.50 ± 0.03 | 1.98 ± 0.01 | 24.24 | 2.0 ± 0.05 |
| C | 5.33 ± 0.09 | 12 ± 0.100 | 18 ± 0.92 | 60 ± 2.11 | 20 ± 0.31 | 20 ± 0.41 | 1.60 ± 0.01 | 2.02 ± 0.02 | 20 | 0.10 ± 0.004 |
| ICF | 8.41± 0.12 | 1.0 ± 0.010 | 30 ± 0.91 | 80 ± 3.00 | 10 ± 0.29 | 10 ± 0.71 | 1.75 ± 0.03 | 2.21 ± 0.02 | 20.45 | 0.25 ± 0.01 |
| ICI | 8.37± 0.08 | 0.55 ± 0.010 | 29 ± 1.01 | 79 ± 1.99 | 11± 0.31 | 10 ± 0.11 | 1.80 ± 0.02 | 2.18 ± 0.01 | 18.09 | 0.5 ± 0.01 |
| ANF | 7.30± 0.07 | 2.5 ± 0.10 | 32 ± 1.21 | 58 ± 2.00 | 12 ± 0.25 | 30 ± 0.21 | 1.51 ± 0.01 | 1.99 ± 0.03 | 24.12 | 2.4 ± 0.09 |
| ANI | 7.21± 0.05 | 1.5 ± 0.019 | 33 ± 1.01 | 57 ± 1.89 | 13 ± 0.08 | 30 ± 0.31 | 1.49 ± 0.03 | 2.01 ± 0.02 | 25.87 | 2.3 ± 0.12 |

T is control, C is contaminated, ICF is cation exchange after treatment, ICI is cation exchange after one year of treatment, ATF is natural attenuation after treatment and ATI is natural attenuation after one year of treatment. The porosity calculatation it was from the averages of DA and DR, by the equation% Po = 100 ((DR-DA) / DR).

Based on the results of table 5, the pH of soil is passed from neutral to acid classification when contaminated with congenital waters, when remediating, both treatments favor the recovery of this indicator, however, the classification of the soil remediated by Cationic exchange is moderatily alkaline and remains so after one year of weathering since no significant differences were observed when comparing them (p>0.05) and may be related to the aggregate CaO; in contrast, the soil treated by attenuation presents a neutral pH in fresh and these conditions remain after weathering, when comparing these two conditions with that of the control soil, no significant differences were observed (p>0.05), so it can be deduced that AN recovers this value to conditions similar to the initial ones; some investigations mention that the spillage of acidic or alkaline materials contribute to the variation of pH and  CE, also affirms that these factors are determinants in the disposition of some nutrients and, although some species are usually tolerant, their development or growth can be affected, in the case of pH, but there may also be metal disposition, mainly to acidic conditions (Morales Bautista et al., 2011; Paz et al., 2014).

In this sense, the CE values ​​were also contrasted, it was found that the control soil has negligible salinity effects, but when contaminated, the classification changes to saline soil (CE of 12 dS·m-1), when comparing the variations of this property after treating, it was obtained that this parameter decreases to conditions less than 4 dS· m-1 in both cases, considering them without salinity problems; However, when contrasting these values ​​with that of the control soil, significant differences were found (p<0.05), for example, treatment by cationic exchange (CI) is 1 dS·m-1 and decreases 0.5 dS·m-1 after of one year of weathering, on the other hand, remediation by natural attenuation (AN) after treatment has a value of 2.5 dS·m-1 and is reduced to a value of 1.5 dS·m-1; In relation to these results, in some restoration work, it has been observed that chemical treatments tend to be faster and more efficient in terms of the removal of an inorganic contaminant due to the compatibility between the treating agent in comparison with the biological agents, the latter they tend to be slower and their efficiency depends on the type of material added since all processes involved are, mainly, metabolic; In the case of common plantings in the Tabasco region, it has been observed that certain grass such as humidicola (Brachiaria humidicola) is tolerant to these levels of salinity and pH, although the nutrients it provides to livestock is very low (Palma-López et al., 2007; Salgado-García et al., 2016). When evaluating others values ​​in contaminated soil with respect to the control, a decrease of approximately 50% CC was observed (with values ​​close to the point of permanent wilting) and, of% MO of ~95%, which coincides with the results of some studies of salinity impacts in which it has been observed that sodium precursors are usually incorporated in the organic matter as well as in hydroxyl ions of the clays, also they can form complexes with the available water, in all cases there is a displacement of H+ ions and, consequently, low water potential and destruction of organic matter is obtained mainly by nutrient entrainment, in the case of pH, it tends to be more acidic, therefore, in places frequently influenced by salts , there are usually reduction effects in Field Capacity (%CC) and Organic Matter (%MO) (Morales et al., 2010; Wang & Peng, 2010; Stoica et al., 2016). When restoring the soil, these properties are recovered according to the type of treatment, in the case of CI, %CC = 30, only ~14% lower compared to the control and this remains constant after one year (%CC = 29). %, p˃0.05), also% MO in these samples increases, the case of ICF is 0.25% and ICI = 0.5%, both well below the value of the control, but a recovery is estimated since the values ​​are higher compared to the contaminated sample that was 0.1%, it is worth mentioning that after the remediation, no organic material was added, but the increase in this property may be related to the removal of sodium, which allowed greater moisture retention and the establishment of some type of vegetation after one year, although it is of great benefit, the values ​​of %MO of the soil treated by CI (very low) did not reach the same classification of the control (medium) soil according to NOM-021-SEMARNAT-2000 (SEMARNAT, 2002; Callejas-Rodríguez et al., 2012).

Similar to the previous, the samples treated by natural attenuation also present recovery in% CC and% MO, for example,% CC in ANF is 32% and ANI of 33%, when comparing them with each other and the control, no differences significant (p˃0.05); however, the values ​​of% MO for ANF and ANI did not differ between them (p˃0.05), but area diferrent with the control (p <0.05) since they are around 50% higher, but they have the same classification as the control according to the aforementioned norm, stands out that 4% of% MO was added and the contaminated soil had 0.1%, so it can be associated that only 1% can remove CE to conditions below the normative limit. Several studies have emphasized the importance of organic matter in soils, most agree that most of the biogeochemical processes in soil depend on them, mainly, the survival and development of plants (López, 2000; García et al., 2010; Jaramillo & Restrepo, 2017), in this sense, natural attenuation has values ​​of %CC and %MO closer to the control, the previous thing recovers importance, due to the fact that in the study area the seeding is practiced by temporary and not by assisted risk (Zavala et al., 2016).

Other properties evaluated were the content of textures, densities and porosity, these parameters are intrinsically related to soil compaction and respiration (González et al., 2009); under this argument, the results of apparent and real density expressed in table 5 were contrasted, it was observed that when polluting, both have a slight increase (DA of ~7% and DR of ~2%) but with significant differences with respect to control (p<0.05), when applying the treatments, cation exchange DA and DR increases ~17% and ~11% respectively in comparison with non-contaminated soil, however, the values ​​obtained for this same variables in natural attenuation decrease and do not they change after of weathering (p>0.05), when comparing these results with those of control soil, no significant differences were found (p> 0.05). In relation to the above, there are studies that mention that changes in soil densities after being contaminated may be related to the formation of higher molecular weight aggregates as compounds are deposited mainly in clays (Suthersan et al., 2016; Marín-García et al., 2016), these effects can be measured in the increase of densities (compaction), decrease in clay content and increase in sands (decrease in field capacity, as well as significant changes in porosity (generally decrease) (Gutiérrez & Zavala, 2002; Khamehchiyan et al., 2007; Maldonado-Chávez et al., 2010); all these investigations can justify the decrease in porosity (% Po), the slight increase in sands (% A) and decrease in clays (% R) in contaminated soil with respect to the control, after the two restoration processes, it can be seen that there are different behaviors, in the case of ICF, there is a change in textures where %A goes from ~ 60% in contaminated to ~ 80% in treated and, thus, remains in ICI (79%, p> 0.05), reflecting in the decrease of% R (from 20% in contaminated to 10% in ICF and ICI, p> 0.05), in the case of% Po, this is lower by ~ 15% after treatment and drops to ~ 25% after one year , it is important to mention that the textural classification in the samples treated by CI is sandy-loam compared to the control that is loam-clay-sandy, also this phenomenon can be related that in the study zone there is forage grass so it does not trampling by livestock is ruled out (Rucks et al., 2004; Zamora et al., 2012). In discrepancy, when evaluating the results of% Po and textures of the treatment by AN, we obtained what% Po recovers to similar values ​​of the control and, although there are significant differences in the textures (p <0.05), the classification coincides with the witness. One of the great challenges of soil impact assessments is to determine the level of effects according to the time of year, mainly in alluvial soils, of which, vertisols or soils with similar properties are the ones that represent the greatest complexity, Palma (2007) reports these soils for the study area, Adams (2008) and Morales (2016) mention that in these soils the effects on porosity may not be as representative, since usually show variations in the expansion and contraction of the clays according to the seasons of the year (autoarado), but factors highlight that factors such as the content of Interchangeable Cations, pH and %MO, are fundamental properties for the establishment, survival and development of the plants, mainly, if the zone is of vocation of agricultural use, reason why the long-term monitoring allows to determine the behavior of the residual components deposited in the a soil matrix, especially, because depending on the environmental conditions it will be the degradation, leaching and disposal of the pollutants (Mulligan et al., 2001; Kavamura & Esposito, 2010).

On the other hand, the results of the metal analyzes performed on the samples of control, contaminated and treated soil are listed in Table 7.

Table 7. Metals present in contaminated and treated soils.

| Soil | Pb | Zn | Ni | Cd | Cu | Cr | Mg | Mn | Na | Ca | K | Fe | V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T | 0.105 | 0.042 | 0.017 | <LD | <LD | <LD | 2.521 | <LD | 2.400 | 5.621 | 0.398 | 4.151 | 0.044 |
| C | 0.131 | 0.054 | 0.019 | <LD | <LD | <LD | 3.852 | <LD | 7.930 | 5.642 | 0.199 | 4.512 | 0.051 |
| ICF | 0.133 | 0.041 | 0.020 | <LD | <LD | <LD | 1.781 | <LD | 2.068 | 9.782 | 0.300 | 4.491 | 0.025 |
| ICI | 0.131 | 0.040 | 0.022 | <LD | <LD | <LD | 1.692 | <LD | 1.988 | 9.113 | 0.298 | 4.497 | 0.021 |
| ANF | 0.129 | 0.052 | 0.021 | <LD | <LD | <LD | 2.801 | <LD | 3.730 | 5.341 | 0.981 | 4.422 | 0.049 |
| ANI | 0.100 | 0.050 | 0.019 | <LD | <LD | <LD | 2.751 | <LD | 3.010 | 4.742 | 0.980 | 4.420 | 0.047 |

CV≤2%, LD: Limit of detection, all Concentration in mg∙Kg-1.

In the results of table 6, it is observed that in all the samples there is absence of Cd, Cr, and Mn, but that, if they found the metals Pb, Zn, Ni. Fe and V, but are below the regulatory limit and, according to some reports, below the tolerance levels for some crops, when comparing the concentrations found in control and contaminated soil, significant differences were found (p <0.05), this may be due to the fact that congenital water also contains these metals (SEMARNAT, 2007; Abioye, 2011; Chambi et al., 2012). Also, these metals coincide with the work of De la Cruz (2012), who conducted a monitoring of nearby soils similar to the study area located in Cárdenas, Tabasco but the reported concentrations by authors, are much larger in comparation with the present work, this may be due to the fact that Cárdenas has greater anthropogenic influence than Cunduacán (study area), although there are no major comparative studies, also can be linked to some studies of atmospheric emissions in the area, which specifies  what impacts for anthropogenic activities they are more ageddate in the Activo Cinco Presidentes (to which Cárdenas belongs) in compared to those of the Activo Samaria (o which Cunduacán belongs), there are also other authors who mention that the metals in soil they can increase for deposition of emissions or effluent carryover in the lower part of the hydrological basin (Pérez-Vidal et al., 2010; Machado et al., 2012).

In order to determine the behavior of these metals, each soil sample was contrasted and it was observed that each one is shared in a different way, for example, there are no significant differences (p>0.05) between the samples of Pb for C, ICF and ICI, but if with T (p<0.05) so that this metal is not removed by the treatment, the rest (Zn, Ni, V and Fe) present differences with T (p<0.05) and with C (p<0.05), but when comparing the values ​​of these metals, no significant differences were found between ICF and ICI (p>0.05), however, V presents discrepancies between these samples (p <0.05) with lower concentrations after weathering (~16%); In contrast, treatment by natural attenuation, shows differences between fresh treated sample (ANF) and weathered sample (ANI) (p<0.05), and these are also lower than the concentration of the contaminated so in this treatment could have mobility of the contaminants; although it should be noted that with the exception of Fe where no significant differences were observed (p>0.05) between the treatments and is 10% higher than control, the rest of the metals in ANI does not present significant differences with respect to the control (p>0.05) . The differences between the metal concentration found between a treatment coincides with the reports that establish that these are more leachable or available at more acidic pH, therefore AN has lower concentrations than CI, the latter may contain stabilized metals in the clays or in micelles formed by Ca(OH)2, however, although this coincides with the literature, it is only a hypothesis since the weathering was carried out in the open air and there is no information on the content of metals in rainfall, in addition to the fact that it is important perform a detailed study of the disposition of the metallic phases according to the variations of the properties during a longer period since some metals such as Pb, V and Fe, present different trends according to the type of treatment (Torres et al., 2012; Reyes et al., 2014).

Like the rest of the metals, the interchangeable cations present in the soil were contrasted, it was observed that when contaminating Mg and Na, compared with the control they increased by 50% and 300% respectively, which can be related to the concentration in the congenital water and that the ions, in the case of the lowering of K, may be part of the organic matter that is lost when contaminated, it was also observed that there is no difference in the concentration of Ca between T and C (p> 0.05) , when treating the soil, IC allows the displacement of the other three cations which are different to the values ​​of the control (p<0.05) (Mg, Na and K are less ~30%, ~Na 14% and K ~25% less than T), instead Ca is greater by ~75% in ICF and reaches up to 64% in ICI, these values ​​coincide with the report of Morales (2011) who establish that these changes are related with the change in pH due to the physical and biological factors of the weather. However, when evaluating the changes in the concentration of these cations in the remediation by natural attenuation, it is observed that Ca in ANF is found in values ​​similar to the control (p> 0.05), but drops to ~ 6% lower than T after weathering, this may also be associated with the decrease of Na that goes from ~55% higher to control in ANF to  25% in ANI, also Mg decreases until reaching values ​​higher than 10% in ANF with respect to the control, this does not vary,  because when comparing with ANI they do not present significant differences (p>0.05), on the other hand potassium that is added in the treating material represents a 240 % greater than the control in ANF but reaches values ​​analogous to T in ANI since They did not find differences between them. For all the previously obtained, AN has higher recoveries and similar values ​​in K and Mg to the control, and Ca and Na do not represent changes between ±25% with respect to the control soil, in contrast, only IC represents values ​​between this range, considering that these factors are also determinants for implementing endemic plants and that they can adapt to these variables after the restoration (Jiménez-Heredia et al., 2010; Ruíz-Álvarez et al., 2011, Trujillo-Narcía et al., 2012).

CONCLUSIONS

In summary and, based on the analysis of the results, we build Table 8, which lists the parameters recovered after weathering the treated soils in comparison with the control, in the case of CE it is considered at values ​​lower than 4 dS·m-1, that CC does not exceed PMP and the Na, Mg, K and Ca cations with estimates between ± 25% with respect to the control, as well as the concentration of the metals Pb, Zn, Ni, Cd, Cu, Cr, Mn, Fe and V, the LMP and tolerance levels for some plants are low.

Table 8. Parameters recovered according to the type of treatment in soil contaminated by congenital waters after one year of weathering.

| Soil | pH | CE | %CC | %MO | DA | DR | %Po | %A | %R | %L | Na | Ca | Mg | K |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IC | Not | Yes | Yes | Not | Not | Not | Not | Not | Not | Not | Yes | Not | Not | Not |
| AN | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |

Therefore, we conclude that when a soil is contaminated, in most cases both the pollutant and the treating agent cause changes in the properties and, these will always be different from those that were originally before a spill, although there are many Environmental impact assessment reports, there are few studies that make comparisons of treatment on the same site, which would save on operating expenses and restore the conditions of use vocation with endemic plants or that were planted before the site had affectations, in this sense, as observed in Table 8, in the long term and, according to some fertility properties, the best treatment would be natural attenuation; Although it must be considered that metals are in very low concentrations, it is recommended to evaluate leachates and metals in plants in order to reduce health risks.

ACKNOWLEDGEMENTS

The National Council of Science and Technology in México is thanked for the postgraduate scholarship assigned to student, also the Academic Division of Basic Sciences of the Autonomous University of Tabasco for the support in the implementation of infrastructure to the Soil and Hydrocarbons Analysis Laboratory, likewise the Autonomous University of Coahuila and the Autonomous University of San Luis Potosí for the analytical determinations made strengthening the formation of human resources and collaboration networks.

REFERENCES

Abioye, O. P. 2011. Biological remediation of hydrocarbon and heavy metals contaminated soil. In Soil contamination. IntechOpen. doi: 10.5772/24938. (Published: September 12th 2011).

Adams, R. H., F. G. Osorio, J. Z. Cruz. 2008. Water repellency in oil contaminated sandy and clayey soils. International Journal of Environmental Science & Technology, 5(4), 445-454. doi: 10.1007/BF03326040. (Published: 01 September 2008).

Adams, R. H., J. Zavala-Cruz y F. A. Morales García. 2008. Concentración residual de hidrocarburos en suelo del trópico. II: afectación a la fertilidad y su recuperación. Interciencia, 33(7), 483-489.

Arao, T., S. Ishikawa, M. Murakami, K. Abe, Y. Maejima, & T. Makino. 2010. Heavy metal contamination of agricultural soil and countermeasures in Japan. Paddy and water Environment, 8(3), 247-257. doi: 10.1007/s10333-010-0205-7. (Published: 18 April 2010).

Azetsu-Scott, K., P. Yeats, G. Wohlgeschaffen, J. Dalziel, S. Niven, & K. Lee. 2007. Precipitation of heavy metals in produced water: Influence on contaminant transport and toxicity. Marine environmental research, 63(2), 146-167. doi: 10.1016/j.marenvres.2006.08.001. (Published: March 2007).

Becerra-Castro, C., A. R. Lopes, I. Vaz-Moreira, E. F. Silva, C. M. Manaia, y O. C. Nunes. 2015. Wastewater reuse in irrigation: A microbiological perspective on implications in soil fertility and human and environmental health. Environment international, 75, 117-135. doi: 10.1016/j.envint.2014.11.001. (Published: February 2015).

Bolan, N., A. Kunhikrishnan, R. Thangarajan, J. Kumpiene, J. Park, T. Makino & K. Scheckel. 2014. Remediation of heavy metal (loid) s contaminated soils–to mobilize or to immobilize?. Journal of Hazardous Materials, 266, 141-166. doi: 10.1016/j.jhazmat.2013.12.018. (Published: 15 February 2014).

Boqué, R., & A. Maroto. 2004. El análisis de la varianza (ANOVA) 1. Comparación de múltiples poblaciones. Grupo de Quimiometrıa y Calimetrıa. Universitat Rovira i Virgil.

Callejas-Rodríguez, R., E. Rojo-Torres, C. Benavidez-Zabala & E. Kania-Kuhl, E. 2012. Crecimiento y distribución de raíces y su relación con el potencial productivo de parrales de vides de mesa. Agrociencia, 46(1), 23-35.

Cavazos-Arroyo, J., B. Pérez-Armendáriz, y A. Mauricio-Gutiérrez. 2014. Afectaciones y consecuencias de los derrames de hidrocarburos en suelos agrícolas de Acatzingo, Puebla, México. Agricultura, sociedad y desarrollo, 11(4), 539-550.

Chambi, L. J., V. Orsag & A. Niura. 2012. Evaluación de la Presencia de metales pesados y arsénico en suelos agrícolas y cultivos en tres micro-cuencas del Municipio de Poopo. Revista Boliviana de Química, 29(1), 111-119.

CNA. 2019. Comisión Nacional del Agua. Resúmenes Mensuales de Temperaturas y Lluvia. https://smn.cna.gob.mx/es/climatologia/temperaturas-y-lluvias/resumenes-mensuales-de-temperaturas-y-lluvias.

Cuero, R. 2013. Hacia un sistema complementario de producción más limpia en suelos degradados por salinidad. Ambiente y Sostenibilidad, 2, 59-68.

Dastgheib, S. M. M., M. A. Amoozegar, K. Khajeh, & A. Ventosa. 2011. A halotolerant Alcanivorax sp. strain with potential application in saline soil remediation. Applied microbiology and biotechnology, 90(1), 305-312. doi:10.1007/s00253-010-3049-6. (Published: 14 December 2010).

De la Cruz-Pons, A., J. Zavala-Cruz, A. Guerrero-Peña, S. Salgado-García, L. C. Lagunes-Espinoza & F. Gavi-Reyes, F. 2012. Metales pesados en suelos cultivados con caña de azúcar en la Chontalpa Tabasco. Universidad y ciencia, 28(2), 119-130.

Diya’uddeen, B. H., W. M. A. W. Daud, & A. A. Aziz. 2011. Treatment technologies for petroleum refinery effluents: A review. Process safety and environmental protection, 89(2), 95-105. doi: 10.1016/j.psep.2010.11.003. (Published: March 2011).

Ebuehi, O. A. T., I. B. Abibo, P. D. Shekwolo, K. I. Sigismund, A. Adoki & I. C. Okoro. 2005. Remediation of crude oil contaminated soil by enhanced natural attenuation technique.

Enamorado, T., L. F. López-Calva, C. Rodríguez-Castelán & H. Winkler. 2016. Income inequality and violent crime: Evidence from Mexico's drug war. Journal of Development Economics, 120, 128-143. doi: 10.1016/j.jdeveco.2015.12.004. (Published: May 2016).

Etim, E. E. 2012. Phytoremediation and its mechanisms: a review. Int J Environ Bioenergy, 2(3), 120-136.

Fawzy, E. M. 2008. Soil remediation using in situ immobilisation techniques. Chemistry and Ecology, 24(2), 147-156. doi: 10.1080/02757540801920154. (Published: 18 April 2008).

García, A., M. Dorado, I. Pérez & E. Montilla, E. 2010. Efecto del déficit hídrico sobre la distribución de fotoasimilados en plantas de arroz (Orysa sativa L.). Interciencia, 35(1).

González Cueto, O., C. E. Iglesias Coronel y M. Herrera Suárez. 2009. Análisis de los factores que provocan compactación del suelo agrícola. Revista Ciencias Técnicas Agropecuarias, 18(2), 57-63.

Gutiérrez, M. D. C., & J. Zavala. 2002. Rasgos hidromórficos de suelos tropicales contaminados con hidrocarburos. Terra Latinoamericana, 20(2).

Hobman, J. L., & L. C. Crossman. 2015. Bacterial antimicrobial metal ion resistance. Journal of medical microbiology, 64, 471-497. doi: 10.1099/jmm.0.023036-0. (Published: 11 February 2016).

Hou, D., & A. Al-Tabbaa. 2014. Sustainability: a new imperative in contaminated land remediation. Environmental Science & Policy, 39, 25-34. doi: 10.1016/j.envsci.2014.02.003. (Published: May 2014).

Hurtado, M. J. R., & V. B. Silvente. 2012. Cómo aplicar las pruebas paramétricas bivariadas t de Student y ANOVA en SPSS. Caso práctico. REIRE, 5(2), 83-100. doi:10.1344/reire2012.5.2527. (Published: July 2012).

Jaramillo, M., & I. Restrepo. 2017. Wastewater reuse in agriculture: A review about its limitations and benefits. Sustainability, 9(10), 1734. doi: 10.3390/su9101734. (Published: 11 October 2017).

Jayasekera, S., & S. Hall. 2007. Modification of the properties of salt affected soils using electrochemical treatments. Geotechnical and Geological Engineering, 25(1), 1. doi: 10.1007/s10706-006-0001-8. (Published: 23 September 2006).

Jiménez-Heredia, Y., C. M. Martínez-Bravo & N. J. Mancera-Rodríguez. 2010. Características físicas y químicas del suelo en diferentes sistemas de uso y manejo en el Centro agropecuario Cotové, Santa Fé de Antioquia. Colombia. Rev. Suelos Ecuat, 40, 176-188.

Kadali, K. K., K. L. Simons, P. P. Skuza, R. B. Moore & A. S. Ball. 2012. A complementary approach to identifying and assessing the remediation potential of hydrocarbonoclastic bacteria. Journal of microbiological methods, 88(3), 348-355. doi: 10.1016/j.mimet.2011.12.006. (Published: March 2012).

Kavamura, V. N., & E. Esposito. 2010. Biotechnological strategies applied to the decontamination of soils polluted with heavy metals. Biotechnology advances, 28(1), 61-69. doi: 10.1016/j.biotechadv.2009.09.002. (Published: February 2010).

Khamehchiyan, M., A. H. Charkhabi & M. Tajik. 2007. Effects of crude oil contamination on geotechnical properties of clayey and sandy soils. Engineering geology, 89(3-4), 220-229. doi: 10.1016/j.enggeo.2006.10.009. (Published: February 2007).

Khodaveisi, J., H. Banejad, A. Afkhami, E. Olyaie, S. Lashgari y R. Dashti. 2011. Synthesis of calcium peroxide nanoparticles as an innovative reagent for in situ chemical oxidation. Journal of hazardous materials, 192(3), 1437-1440. doi: 10.1016/j.jhazmat.2011.06.060. (Published: September 2011).

Labra-Cardón, D., L. A. Guerrero-Zúñiga, A. V. Rodriguez-Tovar, S. Montes-Villafán, S. Pérez-Jiménez y A. Rodríguez-Dorantes. 2012. Respuesta de crecimiento y tolerancia a metales pesados de Cyperus elegans y Echinochloa polystachya inoculadas con una rizobacteria aislada de un suelo contaminado con hidrocarburos derivados del petróleo. Revista internacional de contaminación ambiental, 28(1), 7-16.

Lakhdar, A., M. Rabhi, T. Ghnaya, F. Montemurro, N. Jedidi & C. Abdelly. 2009. Effectiveness of compost use in salt-affected soil. Journal of hazardous materials, 171(1-3), 29-37. doi: 10.1016/j.jhazmat.2009.05.132. (Published: 15 November 2009).

Lázaro Sánchez, I. (2017). La implementación de la reforma energética en México.: Análisis jurídico de la ronda uno de licitaciones para la exploración y extracción de hidrocarburos. Revista Jurídica Derecho, 6(7), 115-136.

Li, X., X. Wang, Y. Zhang, Q. Zhao, B. Yu, Y. Li & Q. Zhou. 2016. Salinity and conductivity amendment of soil enhanced the bioelectrochemical degradation of petroleum hydrocarbons. Scientific reports, 6, 32861. doi: 10.1038/srep32861. (Published: 06 September 2016).

Liang, Y., J. D. Van Nostrand, Y. Deng, Z. He, L. Wu, X. Zhang & J. Zhou. 2011. Functional gene diversity of soil microbial communities from five oil-contaminated fields in China. The ISME journal, 5(3), 403. doi: 10.1038/ismej.2010.142. (Published: 23 September 2010).

López, F. 2000. Relaciones hídricas en el continuo agua-suelo-planta-atmosfera. Universidad Nacional de Colombia-Sede Palmira-Facultad de Ciencias Agropecuarias.

Lu, S., X. Zhang y Y. Xue.  2017. Application of calcium peroxide in water and soil treatment: a review. Journal of hazardous materials, 337, 163-177. doi: 10.1016/j.jhazmat.2017.04.064. (Published: 5 September 2007).

Machado, A., N. García, C. García, L. Acosta, A. Córdova, M. Linares & H. Velásquez. 2008. Contaminación por metales (Pb, Zn, Ni y Cr) en aire, sedimentos viales y suelo en una zona de alto tráfico vehicular. Revista internacional de contaminación ambiental, 24(4), 171-182.

Maldonado-Chávez, E., M. C. Rivera-Cruz, F. Izquierdo-Reyes & D. J. Palma-López. 2010. Efectos de rizósfera, microorganismos y fertilización en la biorremediación y fitorremediación de suelos con petróleos crudo nuevo e intemperizado. Universidad y ciencia, 26(2), 121-136.

Maldonado-Sánchez, E. A., S. Ochoa-Gaona, R. Ramos-Reyes, M. D. L. Á. Guadarrama-Olivera, N. González-Valdivia & B. H. de Jong. 2016. La selva inundable de canacoite en Tabasco, México, una comunidad vegetal amenazada. Acta botánica mexicana, (115), 75-101.

Marín-García, D. C., R. H. Adams & R. Hernández-Barajas. 2016. Effect of crude petroleum on water repellency in a clayey alluvial soil. International journal of environmental science and technology, 13(1), 55-64. doi: 10.1007/s13762-015-0838-6. (Published: 23 July 2015).

Mendoza Cantú, A., & I. A. R. Ize Lema. 2017. Las sustancias químicas en México. perspectivas para un manejo adecuado. Revista internacional de contaminación ambiental, 33(4), 719-745.

Morales, D., J. Dell'Amico, P. Rodríguez, A. Torrecillas & M. D. J. Sánchez-Blanco. 2010. Efecto del estrés por NaCl en el crecimiento y las relaciones hídricas en plantas de tomate (Solanum lycopersicum L.) durante el período vegetativo. Cultivos tropicales, 31(4), 00-00.

Morales-Bautista, C. M., C. E. Lobato García, C. Méndez-Olán y M. D. J. Alor Chávez. 2016. Evaluación del tratamiento del intercambio catiónico en dos suelos aluviales contaminados con aguas congénitas. Interciencia, 41(10), 696-702.

Morales-Bautista, C., V. Domínguez-Rodríguez y R. H. Adams. 2011. Estudio cinético del intercambio catiónico con Ca (OH) 2 y evaluación de la fertilidad en un suelo arcilloso contaminado con aguas congénitas. Bioagro, 23(2), 129-134.

Mulligan, C. N., R. N. Yong & B. F. Gibbs. 2001. Remediation technologies for metal-contaminated soils and groundwater: an evaluation. Engineering geology, 60(1-4), 193-207. doi: 10.1016/S0013-7952(00)00101-0. (Published: June 2001).

Ordóñez, A. D. L. y R. H. A. Schroeder. 2014. Influencia de la geomorfología en la dispersión de hidrocarburos en caso de fuga en ductos del bordo derecho del Campo Samaria. Kuxulkab', 17(33), 55-59.

Ortiz-Salinas, R., S. Cram y I. Sommer. 2012. Hidrocarburos aromáticos policíclicos (HAPs) en suelos de la llanura aluvial baja del estado de Tabasco, México. Universidad y Ciencia, 28(2), 131-144.

Oswald, Ú. 2017. Seguridad, disponibilidad y sustentabilidad energética en México. Revista mexicana de ciencias políticas y sociales, 62(230), 155-195.

Palma-López, D. J., D. J. Cisneros, C. E. Moreno y J. A. Rincón-Ramírez. 2007. Suelos de Tabasco: su uso y manejo sustentable. Colegio de Postgraduados-ISPROTAB-FUPROTAB. Villahermosa, Tabasco, México.

Paz, R. C., R. A. Rocco, J. F. Jiménez-Bremont, M. Rodríguez-Kessler, A. Becerra-Flora, A. B. Menéndez & O. A. Ruíz. 2014. Identification of differentially expressed genes potentially involved in the tolerance of Lotus tenuis to long-term alkaline stress. Plant physiology and biochemistry, 82, 279-288. doi: 10.1016/j.plaphy.2014.06.009. (Published: Septermber 2014).

Peng, J. F., Y. H. Song, P. Yuan, X. Y.  Cui & G. L. Qiu. 2009. The remediation of heavy metals contaminated sediment. Journal of hazardous materials, 161(2-3), 633-640.

Pérez-Vidal, H., M. A. Lunagómez-Rocha & L. Acosta-Pérez. 2010. Análisis de partículas suspendidas totales (PST) y partículas fracción respirable (PM10), en Cunduacán, Tabasco. Universidad y ciencia, 26(2), 151-162.

Pinkus-Rendón, M. J., & A. Contreras-Sánchez. 2012. Impacto socioambiental de la industria petrolera en Tabasco: el caso de la Chontalpa. LiminaR, 10(2), 122-144.

Porta Casanellas, J., M. López-Acevedo Reguerín & C. Roquero de Laburu. 2003. Edafología: para la agricultura y el medio ambiente.

Prieto Méndez, J., C. A. González Ramírez, A. D. Román Gutiérrez y F. Prieto García. 2009. Contaminación y fitotoxicidad en plantas por metales pesados provenientes de suelos y agua. Tropical and Subtropical Agroecosystems, 10(1), 29-44.

Qin, X., J. C. Tang, D. S. Li & Q. M. Zhang. 2012. Effect of salinity on the bioremediation of petroleum hydrocarbons in a saline‐alkaline soil. Letters in applied microbiology, 55(3), 210-217. doi: 10.1111/j.1472-765X.2012.03280. x. (Published: 23 June 2012).

Reyes Rodríguez, R., G. Pierre, F. Guridi Izquierdo & R. Valdés Carmenate. 2014. Disponibilidad de metales pesados en suelos Ferralíticos con baja actividad antrópica en San José de las Lajas, Mayabeque. Revista Ciencias Técnicas Agropecuarias, 23(3), 37-40.

Riojas-Rodríguez, H., A. Schilmann, L. López-Carrillo & J. Finkelman, J. 2013. La salud ambiental en México: situación actual y perspectivas futuras. salud pública de méxico, 55(6), 640-649.

Romo, D. (2016). Refinación de petróleo en México y perspectiva de la reforma energética. Problemas del desarrollo, 47(187), 139-164. doi: 10.1016/j.rpd.2016.10.005. (Published: December 2016).

Rucks, L., F. García, A. Kaplán, J. Ponce de León & M. Hill. 2004. Propiedades físicas del suelo. Universidad de la República: Facultad de agronomía. Montevideo, Uruguay.

Ruíz-Álvarez, O., R. Arteaga-Ramírez, M. A. Vázquez-Peña, R. López-López & R. E. Ontivetos-Capurata. 2011. Requerimiento de riego y predicción del rendimiento en gramíneas forrajeras mediante un modelo de simulación en Tabasco, México. Agrociencia, 45(7), 745-760.

Ruiz-Álvarez, O., R. Arteaga-Ramírez, M. A. Vázquez-Peña, R. E. Ontiveros Capurata & R. López-López. 2012. Balance hídrico y clasificación climática del estado de Tabasco, México. Universidad y ciencia, 28(1), 1-14.

Ruyters, S., J. Mertens, E. Vassilieva, B. Dehandschutter, A. Poffijn & E. Smolders. 2011. The red mud accident in Ajka (Hungary): plant toxicity and trace metal bioavailability in red mud contaminated soil. Environmental science & technology, 45(4), 1616-1622. doi: 10.1021/es104000m. (Published: 4 January 2011).

Saint-Laurent, D., M. Hähni, J. St-Laurent & F. Baril. 2010. Comparative assessment of soil contamination by lead and heavy metals in riparian and agricultural areas (Southern Québec, Canada). International journal of environmental research and public health, 7(8), 3100-3114. doi:10.3390/ijerph7083100. (Published: 5 August 2010).

Salgado-García, S., D.J. Palma-López, J. Zavala-Cruz, S. Córdova-Sánchez, M. Castelán-Estrada, L. D. C. Lagunes-Espinoza y E. Moreno-Caliz. 2016. Programa de fertilización sustentable para plantaciones de cítricos en Tabasco, México. Ecosistemas y recursos agropecuarios, 3(9), 345-356.

SEMARNAT. 2003.  Secretaría de Medio Ambiente y Recursos Naturales. NORMA OFICIAL MEXICANA NOM-143-SEMARNAT-2003, Que establece las especificaciones ambientales para el manejo de agua congénita asociada a hidrocarburos.

SEMARNAT. 2005. Secretaría de Medio Ambiente y Recursos Naturales. Norma Oficial Mexicana NOM-052-SEMARNAT-2005. Que establece las características, el procedimiento de identificación, clasificación y listados de los residuos peligrosos. Diario Oficial de la Federación.

SEMARNAT. 2007. Secretaría de Medio Ambiente y Recursos Naturales. Norma Oficial Mexicana NOM-147-SEMARNAT/SSA1-2004, Que establece criterios para determinar las concentraciones de remediación de suelos contaminados por arsénico, bario, berilio, cadmio, cromo hexavalente, mercurio, níquel, plata, plomo, selenio, talio y/o vanadio. Diario Oficial.

SEMARNAT. 1996. Secretaría de Medio Ambiente y Recursos Naturales. Norma Oficial Mexicana NOM-001-SEMARNAT-1996. Que establece los límites máximos permisibles de contaminantes en las descargas de aguas residuales en aguas y bienes nacionales. Diario Oficial de la Federación.

SEMARNAT. 2002. Secretaría de Medio Ambiente y Recursos Naturales. NOM-021-SEMARNAT-2000. Establece los límites máximos permisibles de contaminantes en las descargas de aguas residuales a cuerpos receptores provenientes de la industria de la destilería. Diario Oficial de la Federación. México, DF.

SEMARNAT. 2003. Secretaría de Medio Ambiente y Recursos Naturales. NOM-053-Semarnat-1993 Que Establece El Procedimiento Para Llevar a Cabo la Prueba De Extracción Para Determinar Los Constituyentes Que Hacen a un Residuo Peligroso por Su Toxicidad al Ambiente.

SEMARNAT. 2003. Secretaría de Medio Ambiente y Recursos Naturales. Norma Oficial Mexicana NOM-143-SEMARNAT-2003, Que establece las especificaciones ambientales para el manejo de agua congénita asociada a hidrocarburos. Publicada en el Diario Oficial de la Federación, el, 3.

Song, B., G. Zeng, J. Gong, J. Liang, P. Xu, Z Liu & S. Ye. 2017. Evaluation methods for assessing effectiveness of in situ remediation of soil and sediment contaminated with organic pollutants and heavy metals. Environment international, 105, 43-55. doi: 10.1016/j.envint.2017.05.001. (Published: August 2017).

Stoica, M. E., L. A. Z. A. R. Avram, I. Onutu, A. Barbulescu, C. Panaitescu, & T. U. D. O. R. A. Cristescu. 2016. Time behaviour of hydrocarbon pollutants in soils polluted with oil and salt water. Revista de Chimie, 67(2), 357-361.

Surendra, K. C., D. Takara, A. G. Hashimoto & S. K. Khanal. 2014. Biogas as a sustainable energy source for developing countries: Opportunities and challenges. Renewable and Sustainable Energy Reviews, 31, 846-859. doi: 10.1016/j.rser.2013.12.015. (Published: March 2014).

Suthersan, S. S., J. Horst, M. Schnobrich, N. Welty & J. McDonough, J. 2016. Remediation engineering: design concepts. CRC Press. doi: 10.1201/9781315367088. (Published: 25 November 2016).

Tang, J., X. Lu, Q. Sun & W. Zhu. 2012. Aging effect of petroleum hydrocarbons in soil under different attenuation conditions. Agriculture, Ecosystems & Environment, 149, 109-117. doi: 10.1016/j.agee.2011.12.020. (Published: March 2012).

Torres, L. G., R. B. Lopez & M. Beltran. 2012. Removal of As, Cd, Cu, Ni, Pb, and Zn from a highly contaminated industrial soil using surfactant enhanced soil washing. Physics and Chemistry of the Earth, Parts A/B/C, 37, 30-36. doi: 10.1016/j.pce.2011.02.003.

Trujillo-Narcía, A., M. D. C. Rivera-Cruz, L. D. C. Lagunes-Espinoza, D. J. Palma-López, S. Soto-Sánchez & G. Ramírez-Valverde. 2012. Efecto de la restauración de un fluvisol contaminado con petróleo crudo. Revista internacional de contaminación ambiental, 28(4), 360-374.

Verma, R., & P. Dwivedi. 2013. Heavy metal water pollution-A case study. Recent Research in Science and Technology, 5(5).

Wang, S. & Y. Peng. 2010. Natural zeolites as effective adsorbents in water and wastewater treatment. Chemical Engineering Journal, 156(1), 11-24. doi: 10.1016/j.cej.2009.10.029. (Published: 1 January 2010).

Yao, Z., J. Li, H. Xie & C. Yu. 2012. Review on remediation technologies of soil contaminated by heavy metals. Procedia Environmental Sciences, 16, 722-729. doi: 10.1016/j.proenv.2012.10.099.

Yongming, L. (2009). Current Research and Development in Soil Remediation Technologies [J]. Progress in Chemistry Z, 1.

Zamora, A., J. Ramos & M. Arias. 2012. Efecto de la contaminación por hidrocarburos sobre algunas propiedades químicas y microbiológicas de un suelo de sabana. Bioagro, 24(1), 5-12.

Zavala-Cruz, J., R. Jiménez Ramírez, D. J. Palma-López, F. Bautista Zúñiga y F. Gavi Reyes. 2016. Paisajes geomorfológicos: base para el levantamiento de suelos en Tabasco, México. Ecosistemas y recursos agropecuarios, 3(8), 161-171.